# CMPG325 Computer Networks

## Final Technical Report

Project Title: Network Design, Implementation and Testing for Botshelo Safari Adventures

Project ID: CMPG325-2026-058

Client ID: CLI-058

Organisation: Botshelo Safari Adventures

Industry: Tourism

Location: Rustenburg, South Africa

Student Name: MH MATILO

Student Number: 39690415

Module: CMPG325 — Computer Networks

Submission: Final Project Submission

Submission Date: 16 October 2026

### Assigned Technical Challenge

Network Address Translation (NAT) — inside/outside address translation.

### Additional Requirements

- Authorised remote management of network devices by an off-site IT contractor.

- Integration of a secondary Internet connection in accordance with Change Request CR15.

## 1. Executive Summary

This project involves the design, implementation and testing of a computer network for Botshelo Safari Adventures, a tourism organisation based in Rustenburg, South Africa. The project was completed as part of the CMPG325 Computer Networks module.

The network was designed using Cisco Packet Tracer and includes multiple VLANs to separate administrative, bookings, finance, management, operations, server, network-management and guest traffic. Inter-VLAN routing is provided by a router-on-a-stick configuration, with separate IP subnets and default gateways assigned to the VLANs.

The primary technical challenge was Network Address Translation (NAT), which allows traffic from the internal network to be translated when accessing simulated external networks. NAT overload was configured on the primary and secondary ISP-facing interfaces. A second ISP connection was also implemented to address Change Request CR15, with a floating static default route configured to provide an alternative path when the primary connection becomes unavailable.

Connectivity tests demonstrated successful access to the simulated external destinations through the primary connection and through the secondary connection during failover testing. The routing table was checked to confirm the change in the default route, and NAT statistics were inspected to verify that traffic was processed by the NAT configuration.

The final submission includes the Cisco Packet Tracer project, a GitHub portfolio containing the project and supporting evidence, a technical report, and a planned video demonstration. The remote-management requirement will be documented according to the configuration and evidence verified in the final project.

## 2. Introduction and Project Objectives

### 2.1 Introduction

Botshelo Safari Adventures is a tourism organisation based in Rustenburg, South Africa. The organisation requires a structured computer network to support its business operations and provide connectivity between different departments and network resources.

This project forms part of the CMPG325 Computer Networks module. The network was designed and implemented in Cisco Packet Tracer to demonstrate practical networking concepts, including Virtual Local Area Networks (VLANs), inter-VLAN routing, IP addressing, Network Address Translation (NAT), and Internet connection redundancy.

The network uses VLANs to separate traffic for the Administration, Bookings, Finance, Management, Operations, Servers, Network Management and Guest departments or functions. Each VLAN has its own IP subnet and default gateway. Inter-VLAN routing is provided through a router-on-a-stick configuration, allowing communication between VLANs where routing and access policies permit it.

The main technical challenge assigned to the project was implementing NAT to translate internal network traffic when communicating with simulated external networks. In addition, Change Request CR15 required a second Internet Service Provider (ISP) connection to improve connectivity resilience. A primary default route and a floating static default route were configured to provide an alternative route when the primary ISP connection becomes unavailable.

The project also includes a requirement for authorised remote management of network devices by an off-site IT contractor. This requirement must be assessed against the final device configuration and supporting evidence to establish whether it has been fully implemented and tested.

### 2.2 Project Aim

The aim of this project is to design, configure and test a functional and logically segmented network for Botshelo Safari Adventures using Cisco Packet Tracer, while addressing the assigned NAT challenge and the additional requirement for a secondary ISP connection.

### 2.3 Project Objectives

The main objectives of the project are to:

- Design a network topology that connects the organisation's network devices, end-user computers, server and external ISP networks.

- Divide the internal network into eight VLANs to separate departmental, server, guest and network-management traffic.

- Develop and implement an IP addressing plan that assigns suitable subnets, subnet masks and default gateways to the VLANs.

- Configure trunk links between the core switch and access switches to carry traffic for the required VLANs.

- Configure inter-VLAN routing so that communication between different VLANs can occur where required.

- Implement NAT overload on the ISP-facing interfaces to translate traffic from the internal network to the simulated external destinations.

- Configure a primary ISP connection and a secondary ISP connection to meet Change Request CR15.

- Configure and test a floating static default route so that the secondary connection can provide an alternative path when the primary connection fails.

- Test network connectivity, routing behaviour and NAT operation, and record the results as evidence.

- Address the requirement for secure, authorised remote management of network devices by an off-site IT contractor, subject to verification of the final configuration.

- Document the design, implementation, testing process, limitations and recommendations in a technical report and GitHub portfolio.

### 2.4 Project Scope

The project covers the design and simulation of the internal network, VLAN configuration, IP addressing, inter-VLAN routing, NAT, ISP connectivity, routing redundancy and connectivity testing. It also considers remote device management as a project requirement.

The implementation is simulated in Cisco Packet Tracer. Consequently, the results demonstrate the behaviour of the configured simulated network and do not, by themselves, establish the performance or security of a live production network.

### 2.5 Expected Outcomes

The expected outcome is a documented and tested Packet Tracer network that provides logical separation of internal traffic, routing between VLANs, NAT-based access to simulated external destinations, and an alternative ISP route during a primary-link failure. The final submission should include the Packet Tracer file, technical report, GitHub evidence and video demonstration required by the project brief.

## 3. Client Requirements and Design Constraints

### 3.1 Client Requirements

The network solution was developed to meet the networking needs of Botshelo Safari Adventures. The main requirements addressed by the project are described below.

3.1.1 Network segmentation

The internal network is divided into eight VLANs: Administration (VLAN 10), Bookings (VLAN 20), Finance (VLAN 30), Management (VLAN 40), Operations (VLAN 50), Servers (VLAN 60), Network Management (VLAN 70) and Guest (VLAN 80). This logical separation helps organise network resources and limits unnecessary mixing of traffic between different functions.

3.1.2 IP addressing

Each VLAN is assigned a separate IP subnet, subnet mask and default gateway. This addressing plan supports communication within each subnet and enables routing between VLANs through the router-on-a-stick configuration.

3.1.3 Inter-VLAN routing

The network must support routing between VLANs where communication is required. The router uses subinterfaces associated with the relevant VLANs to provide their default gateways.

3.1.4 Network Address Translation

The assigned technical challenge is Network Address Translation (NAT). NAT overload is configured on the ISP-facing interfaces to translate traffic originating from the internal network when it accesses the simulated external destinations.

3.1.5 Secondary ISP connection — CR15

Change Request CR15 requires a second ISP connection to improve resilience. The design includes a primary ISP link and a secondary ISP link. A floating static default route provides an alternative path when the primary route is unavailable.

3.1.6 Remote network management

The project brief requires network devices to support authorised remote management by an off-site IT contractor. The final configuration should be checked for suitable remote-access settings and access restrictions. This report will record the implementation and testing status based on the evidence available in the final Packet Tracer project.

3.1.7 Testing and documentation

The completed solution must be tested, and the results documented. Evidence should demonstrate connectivity, NAT operation and the change in routing behaviour when the primary ISP link is unavailable. The submission also includes the Packet Tracer project, GitHub portfolio, technical report and video demonstration.

### 3.2 Design Constraints

The following constraints apply to the project:

- Assigned address block: The network must use the assigned address block, 172.30.34.0/23, for internal addressing.

- Simulation environment: The network is implemented in Cisco Packet Tracer, so testing is limited to the features and behaviours supported by the simulation.

- Assigned challenge: NAT is the required technical challenge and must be demonstrated through configuration and testing.

- Change Request CR15: A secondary ISP connection must be included and tested for failover behaviour.

- Remote access: Remote management must be authorised and appropriately restricted; enabling access alone is not sufficient to demonstrate that it is secure.

- Evidence requirements: Configuration and test results must be supported by the Packet Tracer file and relevant screenshots or other evidence.

- Report accuracy: The report must distinguish between features that have been configured and tested, features that are only partially implemented, and requirements that still need verification.

### 3.3 Summary

The network design addresses segmentation, addressing, routing, NAT and ISP redundancy. The final assessment must also verify the remote-management requirement and ensure that the evidence submitted supports the claims made about the implementation.

## 4. Physical and Logical Network Topology

### 4.1 Physical Network Topology

The network for Botshelo Safari Adventures was designed and simulated in Cisco Packet Tracer. It consists of a core switch, three access switches, an edge router, end-user devices, a server and two simulated ISP connections.

The core switch, SW-CORE, provides the central switching point for the internal network. It connects to the three access switches through trunk links, allowing traffic for the required VLANs to pass between the switches. The server, SRV-1, is connected to SW-CORE through interface FastEthernet0/4, which is configured as an access port in VLAN 60 (SERVERS).

The edge router, R-EDGE, connects the internal network to the external ISP networks. Its GigabitEthernet0/0 interface supports the router-on-a-stick configuration, using subinterfaces to provide routing for the internal VLANs. GigabitEthernet0/1 connects to the primary ISP, while GigabitEthernet0/2 connects to the secondary ISP.

The primary ISP uses the address 10.0.0.2/30 on its connection to R-EDGE and provides the simulated external destination 203.0.113.1. The secondary ISP uses 10.0.0.6/30 and provides the simulated external destination 198.51.100.1.

### 4.2 Logical Network Topology

The logical topology separates the internal network into eight VLANs. Each VLAN has its own IP subnet and default gateway. The router's sub-interfaces are associated with the relevant VLAN IDs, enabling inter-VLAN routing.

The core and access switches use VLAN trunk links to carry traffic for multiple VLANs. End devices connect through access ports assigned to the appropriate VLAN. This structure provides logical separation while allowing the router to route traffic between subnets when required.

The edge router also performs NAT overload for traffic destined for the simulated external networks. A primary default route directs traffic through the primary ISP. A floating static default route with a higher administrative distance provides a backup path through the secondary ISP if the primary route becomes unavailable.

### 4.3 Network Components

| Component | Function |
| --- | --- |
| SW-CORE | Central switching and connection point for the access switches and server |
| SW-ACCESS1 | Provides access connectivity for end devices assigned to its ports |
| SW-ACCESS2 | Provides access connectivity for end devices assigned to its ports |
| SW-ACCESS3 | Provides access connectivity for end devices assigned to its ports |
| R-EDGE | Provides inter-VLAN routing, NAT and external routing |
| SRV-1 | Server connected to VLAN 60 (SERVERS) |
| Primary ISP1 | Provides the primary simulated external connection |
| Secondary ISP2 | Provides the backup simulated external connection for CR15 |
| End-user PCs | Represent client devices on the internal network |

### 4.4 WAN Addressing

| Connection | Interface | IP address |
| --- | --- | --- |
| R-EDGE to primary ISP | R-EDGE G0/1 | 10.0.0.1/30 |
| Primary ISP connection | ISP1 G0/0 | 10.0.0.2/30 |
| R-EDGE to secondary ISP | R-EDGE G0/2 | 10.0.0.5/30 |
| Secondary ISP connection | ISP2 G0/0 | 10.0.0.6/30 |
| Primary simulated external destination | ISP1 Loopback0 | 203.0.113.1/32 |
| Secondary simulated external destination | ISP2 Loopback0 | 198.51.100.1/32 |

### 4.5 Failover Design

The primary ISP is the preferred route for external traffic. The router has a default route through 10.0.0.2 and a floating static default route through 10.0.0.6 with an administrative distance of 10.

During testing, the primary link was taken down and traffic to the secondary simulated external destination was tested. The routing table was checked to confirm that the secondary route became the active default route. After the primary connection was restored, the preferred route returned to the primary ISP.

This demonstrates the intended routing failover behaviour in the Packet Tracer simulation. It does not automatically prove that every type of upstream ISP failure will be detected; that depends on the failure-detection mechanisms configured in the network.

## 5. VLANs and IP Addressing Plan

### 5.1 Overview

The assigned internal address block for Botshelo Safari Adventures is 172.30.34.0/23. This address block provides 512 total IPv4 addresses across the range 172.30.34.0 to 172.30.35.255, including network and broadcast addresses.

The network was subnetted to support eight VLANs with different address requirements. Four VLANs use a /26 subnet mask, while the remaining four use a /27 subnet mask. A /26 subnet provides 62 usable host addresses, whereas a /27 subnet provides 30 usable host addresses.

Each VLAN has a dedicated subnet and default gateway configured on a router subinterface. This arrangement supports logical separation of departmental traffic and provides the Layer 3 gateway needed for communication beyond the local subnet.

### 5.2 VLAN and Subnet Allocation

| VLAN ID | VLAN name | Network address | Subnet mask | Default gateway |
| --- | --- | --- | --- | --- |
| 10 | ADMIN | 172.30.34.0/26 | 255.255.255.192 | 172.30.34.1 |
| 20 | BOOKINGS | 172.30.34.64/26 | 255.255.255.192 | 172.30.34.65 |
| 50 | OPERATIONS | 172.30.34.128/26 | 255.255.255.192 | 172.30.34.129 |
| 80 | GUEST | 172.30.34.192/26 | 255.255.255.192 | 172.30.34.193 |
| 30 | FINANCE | 172.30.35.0/27 | 255.255.255.224 | 172.30.35.1 |
| 40 | MANAGEMENT | 172.30.35.32/27 | 255.255.255.224 | 172.30.35.33 |
| 60 | SERVERS | 172.30.35.64/27 | 255.255.255.224 | 172.30.35.65 |
| 70 | NET-MGMT | 172.30.35.96/27 | 255.255.255.224 | 172.30.35.97 |

### 5.3 Host Address Ranges

| VLAN | Usable host range | Broadcast address |
| --- | --- | --- |
| ADMIN | 172.30.34.1–172.30.34.62 | 172.30.34.63 |
| BOOKINGS | 172.30.34.65–172.30.34.126 | 172.30.34.127 |
| OPERATIONS | 172.30.34.129–172.30.34.190 | 172.30.34.191 |
| GUEST | 172.30.34.193–172.30.34.254 | 172.30.34.255 |
| FINANCE | 172.30.35.1–172.30.35.30 | 172.30.35.31 |
| MANAGEMENT | 172.30.35.33–172.30.35.62 | 172.30.35.63 |
| SERVERS | 172.30.35.65–172.30.35.94 | 172.30.35.95 |
| NET-MGMT | 172.30.35.97–172.30.35.126 | 172.30.35.127 |

The ranges above show the mathematically usable addresses in each subnet. The first usable address is assigned as the default gateway in this design. Not every usable address is necessarily assigned to a device.

### 5.4 Example Device Addressing

The following table records example device addresses used in the implementation.

| Device | VLAN | IP address | Subnet mask | Default gateway |
| --- | --- | --- | --- | --- |
| PC-ADMIN1 | 10 | 172.30.34.10 | 255.255.255.192 | 172.30.34.1 |
| PC-ADMIN2 | 10 | 172.30.34.11 | 255.255.255.192 | 172.30.34.1 |
| SRV-1 | 60 | 172.30.35.70 | 255.255.255.224 | 172.30.35.65 |

The addresses of the remaining end devices should be documented from the final Packet Tracer configuration to ensure that the report matches the submitted project.

### 5.5 Router-on-a-Stick Configuration

Inter-VLAN routing is implemented on R-EDGE using sub-interfaces on GigabitEthernet0/0. Each subinterface is associated with its VLAN using IEEE 802.1Q encapsulation and is assigned the corresponding default gateway address.

This configuration allows one physical router interface to route traffic for multiple VLANs. The switch port connected to the router must operate as a trunk and carry the required VLANs for the subinterfaces to communicate with the corresponding VLANs.

### 5.6 Addressing Design Considerations

The use of separate subnets makes the network easier to organise and troubleshoot. It also provides a foundation for applying access-control policies between departments, servers, guest devices and network-management resources.

Although VLANs separate Layer 2 broadcast domains, VLAN configuration alone does not prevent all communication between them. Appropriate routing policies and access-control lists may be needed to restrict traffic, particularly for guest access and management interfaces.

The internal addressing plan uses the assigned /23 block without overlapping the listed VLAN subnets. WAN addresses are allocated separately to the simulated ISP links using /30 subnets.

## 6. Network Implementation and Device Configuration

### 6.1 Implementation Environment

The network was implemented and tested using Cisco Packet Tracer. The simulation includes Cisco routers and switches, end-user computers, a server and two simulated ISP networks. Device configurations were applied through the Cisco IOS command-line interface (CLI).

The implementation was completed in stages, beginning with the internal switching infrastructure and IP addressing, followed by routing, NAT configuration and the secondary ISP connection.

### 6.2 VLAN Configuration

Eight VLANs were created to separate the organisation's internal network into logical groups.

| VLAN ID | VLAN name |
| --- | --- |
| 10 | ADMIN |
| 20 | BOOKINGS |
| 30 | FINANCE |
| 40 | MANAGEMENT |
| 50 | OPERATIONS |
| 60 | SERVERS |
| 70 | NET-MGMT |
| 80 | GUEST |

End-device switch ports were assigned to their corresponding VLANs. The server, SRV-1, was connected to SW-CORE interface FastEthernet0/4, configured as an access port in VLAN 60.

### 6.3 Trunk Link Configuration

Trunk links were configured between SW-CORE and the three access switches. The trunk links use IEEE 802.1Q encapsulation and carry the VLANs required by the network design.

The configured trunk interfaces on SW-CORE are FastEthernet0/1, FastEthernet0/2 and FastEthernet0/3. The show interfaces trunk command was used to inspect the trunking status and verify that these interfaces were operating as trunks.

Trunk configuration allows multiple VLANs to traverse a single physical link, reducing the need for separate physical links for each VLAN.

### 6.4 Inter-VLAN Routing

Inter-VLAN routing was implemented on R-EDGE using a router-on-a-stick design. The router's GigabitEthernet0/0 physical interface connects to the switching infrastructure, while its subinterfaces provide Layer 3 gateways for the eight VLANs.

Each subinterface uses an 802.1Q VLAN tag and an IP address corresponding to the gateway in the addressing plan. This configuration allows the router to forward traffic between VLAN subnets when routing and security policies permit it.

Connectivity was checked by testing end-device communication with the relevant default gateways and other network destinations.

### 6.5 WAN Interface Configuration

R-EDGE uses three GigabitEthernet interfaces for internal and external connectivity.

| Router interface | Connection | Address |
| --- | --- | --- |
| GigabitEthernet0/0 | Internal network | VLAN subinterfaces |
| GigabitEthernet0/1 | Primary ISP | 10.0.0.1/30 |
| GigabitEthernet0/2 | Secondary ISP | 10.0.0.5/30 |

The primary ISP interface connects to 10.0.0.2/30, and the secondary ISP interface connects to 10.0.0.6/30. Both WAN interfaces were configured and tested for connectivity to their directly connected ISP routers.

### 6.6 Default Route Configuration

A primary static default route was configured to direct external traffic through the primary ISP:

ip route 0.0.0.0 0.0.0.0 10.0.0.2

A floating static default route was also configured through the secondary ISP with an administrative distance of 10:

ip route 0.0.0.0 0.0.0.0 10.0.0.6 10

The higher administrative distance makes the secondary route less preferred when the primary route is available. If the primary route is removed from the routing table, the secondary route can become active.

### 6.7 Configuration Verification

Cisco IOS verification commands were used during implementation to inspect the device configuration and operational state.

| Command | Purpose |
| --- | --- |
| show vlan brief | Check VLAN creation and access-port assignments |
| show interfaces trunk | Inspect trunk links and VLAN carriage |
| show ip interface brief | Check interface addressing and status |
| show ip route | Inspect connected networks and the active default route |
| show running-config | Inspect the active device configuration |
| show ip nat statistics | Inspect NAT configuration and translation statistics |
| show ip nat translations | Display active NAT translation entries |

The results of these commands, together with connectivity tests and screenshots, provide evidence for the implementation. The final report should include only results that can be confirmed from the submitted Packet Tracer project or saved evidence.

## 7. Network Address Translation (NAT)

### 7.1 Introduction

Network Address Translation (NAT) is a networking technique that modifies IP address information in packet headers as traffic passes between networks. In this project, NAT is used to translate traffic originating from the internal network when communicating with simulated external destinations.

The implementation uses Port Address Translation (PAT), also known as NAT overload. This allows multiple internal devices to share the IP address of an external interface when accessing the simulated external network.

### 7.2 NAT Design

NAT was configured on R-EDGE. The router's internal VLAN subinterfaces were designated as NAT inside interfaces, while the two ISP-facing interfaces were designated as NAT outside interfaces.

The NAT design supports two simulated external destinations:

- Primary ISP destination: 203.0.113.1

- Secondary ISP destination: 198.51.100.1

Separate access control lists (ACLs) identify internal traffic destined for each simulated external destination. NAT overload rules then translate the matching traffic using the IP address of the relevant ISP-facing interface.

### 7.3 NAT Interface Configuration

The internal VLAN subinterfaces on R-EDGE were configured with the ip nat inside command. The external interfaces were configured as follows:

| Interface | Role | NAT designation |
| --- | --- | --- |
| G0/0.10 | VLAN 10 — ADMIN | Inside |
| G0/0.20 | VLAN 20 — BOOKINGS | Inside |
| G0/0.30 | VLAN 30 — FINANCE | Inside |
| G0/0.40 | VLAN 40 — MANAGEMENT | Inside |
| G0/0.50 | VLAN 50 — OPERATIONS | Inside |
| G0/0.60 | VLAN 60 — SERVERS | Inside |
| G0/0.70 | VLAN 70 — NET-MGMT | Inside |
| G0/0.80 | VLAN 80 — GUEST | Inside |
| G0/1 | Primary ISP | Outside |
| G0/2 | Secondary ISP | Outside |

Correct inside and outside interface designations are necessary for the router to identify the direction of NAT processing.

### 7.4 NAT Access Control Lists and Overload Rules

The following configuration identifies traffic from the internal address block destined for the primary and secondary simulated external destinations.

Primary ISP NAT rule

access-list 101 permit ip 172.30.0.0 0.0.255.255 host 203.0.113.1
ip nat inside source list 101 interface GigabitEthernet0/1 overload

Secondary ISP NAT rule

access-list 102 permit ip 172.30.0.0 0.0.255.255 host 198.51.100.1
ip nat inside source list 102 interface GigabitEthernet0/2 overload

The wildcard mask 0.0.255.255 matches addresses within the 172.30.0.0/16 range, which includes the project's assigned internal 172.30.34.0/23 block. The ACLs additionally restrict matching traffic to the respective simulated external destination.

The overload keyword enables multiple internal devices to share the address of the selected external interface by distinguishing their translations using transport-layer identifiers where applicable.

### 7.5 NAT Testing and Results

NAT was tested by generating traffic from an internal client toward the simulated external destinations and inspecting the router's NAT statistics and routing table.

| Test | Expected behaviour | Recorded result |
| --- | --- | --- |
| Internal client to primary destination 203.0.113.1 | Traffic uses the primary ISP and is eligible for translation by ACL 101 | Replies were received in the final test, although the first ping timed out |
| Primary ISP unavailable; client to secondary destination 198.51.100.1 | Traffic uses the secondary ISP and is eligible for translation by ACL 102 | Four out of four ping replies were received during failover testing |
| Inspect routing table during failover | Secondary default route becomes active when the primary route is unavailable | The default route changed to the secondary next hop, 10.0.0.6 |
| Inspect NAT statistics | NAT counters should reflect matching traffic | NAT statistics showed 22 hits and 46 misses at the recorded check |

The first ping to a destination may time out while connectivity or address resolution is being established. Subsequent successful replies provide evidence of connectivity, but ping results alone do not prove that NAT translated the traffic. The NAT configuration and counters should also be examined.

At the time of the recorded check, the number of active NAT translations was zero. This does not necessarily indicate a configuration failure because ICMP translation entries may expire after the traffic stops. Translation entries should be inspected while test traffic is being generated for clearer evidence.

### 7.6 Limitations and Interpretation

The configured NAT rules are destination-specific: ACL 101 matches traffic to 203.0.113.1, while ACL 102 matches traffic to 198.51.100.1. Therefore, the rules demonstrate translation for the two designated simulated destinations rather than unrestricted access to every possible Internet address.

The NAT results must be interpreted alongside routing, interface status and connectivity tests. A successful ping establishes reachability, while NAT counters and active translation entries provide additional evidence that the translation mechanism is operating.

### 7.7 Conclusion

NAT overload was configured on R-EDGE for the primary and secondary simulated ISP connections. The configuration includes separate ACLs, inside and outside interface designations, and overload rules for the two external destinations. Testing demonstrated successful connectivity to the simulated destinations and provided supporting evidence through NAT statistics and routing-table checks.

## 8. Primary and Secondary ISP Configuration and Failover

### 8.1 Introduction

Change Request CR15 requires the network to include a second ISP connection to improve connectivity resilience. The implementation uses two simulated ISP routers connected to R-EDGE. The primary ISP is the preferred path for external traffic, while the secondary ISP provides an alternative path when the primary route becomes unavailable.

The design uses static routing with a floating static default route. This approach provides a simple mechanism for selecting the secondary path when the primary default route is no longer available in the routing table.

### 8.2 ISP Addressing and Connectivity

The two ISP connections use separate /30 subnets, each providing two usable host addresses for the point-to-point link.

| Device | Interface | IP address | Purpose |
| --- | --- | --- | --- |
| R-EDGE | G0/1 | 10.0.0.1/30 | Primary ISP link |
| Primary ISP1 | G0/0 | 10.0.0.2/30 | Primary ISP router connection |
| R-EDGE | G0/2 | 10.0.0.5/30 | Secondary ISP link |
| Secondary ISP2 | G0/0 | 10.0.0.6/30 | Secondary ISP router connection |
| Primary ISP1 | Loopback0 | 203.0.113.1/32 | Simulated primary external destination |
| Secondary ISP2 | Loopback0 | 198.51.100.1/32 | Simulated secondary external destination |

The WAN interfaces were configured and checked for connectivity to the directly connected ISP routers. The recorded tests showed successful replies from both ISP next-hop addresses.

### 8.3 Default Routing Configuration

The primary default route is configured as follows:

ip route 0.0.0.0 0.0.0.0 10.0.0.2

This directs traffic for destinations not otherwise present in the routing table through the primary ISP.

The floating static default route is configured as follows:

ip route 0.0.0.0 0.0.0.0 10.0.0.6 10

The final value, 10, specifies the administrative distance of the secondary route. Because it is higher than the default administrative distance of a static route, the secondary route is less preferred while the primary route remains available.

### 8.4 Failover Testing Procedure

The failover test was performed using the following general procedure:

- Verify that both ISP links are configured and operational.

- Inspect the routing table on R-EDGE to identify the active default route.

- Confirm that the primary route uses next hop 10.0.0.2.

- Disable or disconnect the primary link for the test.

- Inspect the routing table to determine whether the secondary default route becomes active through 10.0.0.6.

- Generate traffic from an internal client to the secondary simulated external destination, 198.51.100.1.

- Restore the primary link and verify that the preferred primary route returns.

### 8.5 Failover Test Results

| Test | Expected result | Recorded outcome |
| --- | --- | --- |
| Test primary ISP next hop | R-EDGE can reach 10.0.0.2 | Replies were received, with one packet lost in the recorded test |
| Test secondary ISP next hop | R-EDGE can reach 10.0.0.6 | Five out of five replies were received |
| Inspect route with primary available | Default route uses 10.0.0.2 | Primary route was active |
| Disable primary link | Secondary default route becomes active if the primary route is withdrawn | Routing table changed to use 10.0.0.6 |
| Test secondary external destination | Internal client can reach 198.51.100.1 through the backup path | Four out of four replies were received |
| Restore primary link | Primary route becomes preferred again | Routing returned to the primary next hop, 10.0.0.2 |

The results demonstrate that the floating static route operated as intended during the simulated primary-link failure. The successful ping to the secondary destination also supports the conclusion that the backup path was usable during the test.

### 8.6 Limitations

A floating static route provides failover when the primary route is removed from the routing table. If the physical interface remains up while a failure occurs farther upstream, a simple static route may remain active unless additional failure detection is configured.

In a production environment, mechanisms such as IP Service Level Agreements (IP SLA) with object tracking, or an appropriate dynamic routing protocol, could improve detection of upstream failures. These mechanisms are recommendations unless they are explicitly configured and demonstrated in the submitted simulation.

### 8.7 Conclusion

The network includes both primary and secondary ISP connections to meet Change Request CR15. A primary default route and a floating static default route were configured, and failover was tested by making the primary link unavailable. The recorded routing-table and connectivity results demonstrate the intended backup-routing behaviour in Cisco Packet Tracer.

## 9. Remote Network Management

### 9.1 Requirement

The project requires network devices to support remote management by an authorised IT contractor working off-site. Remote management enables an administrator to inspect device status, troubleshoot connectivity problems and maintain network configurations without needing to be physically present at the organisation's premises.

Remote access must be configured carefully because management interfaces provide privileged access to network infrastructure.

### 9.2 Recommended Management Approach

Secure Shell (SSH) is the recommended method for remote command-line administration of Cisco network devices. Unlike Telnet, SSH encrypts the management session, helping protect login credentials and administrative commands while they travel across the network.

A suitable remote-management design should include:

- SSH access rather than unencrypted Telnet.

- Local user accounts or an appropriately configured central authentication service.

- Strong administrative passwords and suitable privilege levels.

- Access restrictions so that only authorised management hosts or networks can reach the device management interfaces.

- Management access through the designated network-management segment, VLAN 70 (NET-MGMT), where supported by the final design.

- Verification that remote access works from an authorised management host and is denied to unauthorised hosts where access restrictions are configured.

### 9.3 Implementation Status

Secure remote management was configured on the R-EDGE router using Secure Shell (SSH) version 2. The configuration included setting the hostname to R-EDGE, configuring the domain name botshelosafari.local, creating a local administrator account, generating 1024-bit RSA keys, and restricting the VTY lines to SSH with local authentication.

The SSH status was verified using the show ip ssh command, which confirmed that SSH version 2 was enabled. The running configuration was inspected to confirm the local user account and VTY settings. The configuration was saved to startup configuration.

The network design includes VLAN 70 (NET-MGMT), with subnet 172.30.35.96/27 and default gateway 172.30.35.97. However, the remote-access test was performed from MGMT-PC1 in VLAN 40 (MANAGEMENT), using IP address 172.30.35.40 and the router gateway 172.30.35.33. The PC successfully pinged the gateway, and an SSH session was established with R-EDGE.

Therefore, SSH remote access has been configured and tested from VLAN 40. Dedicated remote-management access through VLAN 70, and access restrictions against unauthorised hosts, still require separate verification.

### 9.4 Recommended Verification

The following checks were performed or identified for verifying the remote-management configuration:

| Verification | Result |
| --- | --- |
| Inspect the running configuration | Confirmed hostname, domain name, local user account, SSH version and VTY settings |
| Check SSH status using show ip ssh | SSH version 2 confirmed enabled |
| Inspect VTY line configuration | login local and transport input ssh confirmed |
| Save the configuration | Successful; [OK] displayed |
| Test connectivity from MGMT-PC1 | Successful ping to 172.30.35.33 |
| Test SSH from MGMT-PC1 | Successful remote session to R-EDGE |
| Test SSH through VLAN 70 (NET-MGMT) | Not yet verified |
| Test access from an unauthorised host | Not yet verified |

The verification results demonstrate that SSH remote access to R-EDGE works from the existing management workstation in VLAN 40. Further testing is required to confirm dedicated VLAN 70 management access and the effectiveness of restrictions against unauthorised hosts.

### 9.5 Security Considerations

Remote-management services should be limited to authorised administrators. SSH should be preferred over Telnet, administrative credentials should be protected, and management access should be restricted to trusted sources wherever possible.

The management VLAN should also be protected by suitable routing and access-control policies. VLAN separation alone does not guarantee that devices in other VLANs cannot access management services.

### 9.6 Conclusion

SSH version 2 was successfully configured on R-EDGE, and remote access was demonstrated from MGMT-PC1 in VLAN 40 (MANAGEMENT). The router's SSH status and relevant VTY configuration were inspected, and the configuration was saved.

Although VLAN 70 (NET-MGMT) is included in the network design, remote access through that dedicated management VLAN has not yet been demonstrated. Similarly, access restrictions against unauthorised hosts have not been tested. The requirement is therefore partially verified: remote SSH administration works from the tested management workstation, while the remaining security and VLAN 70 verification tasks require further testing.

## 10. Testing and Verification

Testing and verification are necessary to determine whether the implemented network configuration meets the project requirements. The tests conducted focused on inter-VLAN connectivity, router interface status, ISP connectivity and secure remote management.

### 10.1 Interface Verification

The show ip interface brief command was used on R-EDGE to inspect the status of the router interfaces. The configured VLAN subinterfaces, GigabitEthernet0/1 and GigabitEthernet0/2 were reported as up/up. This confirmed that the router interfaces were operational at the time of testing.

### 10.2 Connectivity Verification

A connectivity test from MGMT-PC1 to the router's VLAN 40 gateway, 172.30.35.33, returned four replies. This confirmed IP connectivity between the workstation and the router through VLAN 40 (MANAGEMENT).

The primary ISP connection at 10.0.0.2 and secondary ISP connection at 10.0.0.6 had also been tested during configuration. The secondary ISP gateway responded to all five ping attempts, while the primary ISP gateway responded to four of five attempts during the recorded test.

### 10.3 Remote Management Verification

SSH version 2 was enabled on R-EDGE. The show ip ssh command confirmed that SSH version 2 was active. The VTY configuration was inspected and confirmed to use local authentication and accept SSH connections. A remote session to R-EDGE was successfully established from MGMT-PC1.

### 10.4 Testing Limitations

The tests recorded above verify interface status, selected IP connectivity and SSH remote access. They do not, by themselves, prove that all VLANs can communicate as intended, that NAT translations are working correctly for all required traffic, or that automatic ISP failover has been fully demonstrated. These functions should be tested separately, and the results should be supported by Packet Tracer screenshots or command output.

### 10.5 Summary

The recorded results demonstrate that the router interfaces were operational, the tested management workstation could reach its VLAN gateway, and SSH remote access to R-EDGE was successful. Further evidence should be included for NAT operation, the secondary ISP failover behaviour and connectivity across the remaining VLANs wherever these are required by the project rubric.

## 11. Conclusion

This project involved the design, implementation and testing of a computer network for Botshelo Safari Adventures, a tourism organisation based in Rustenburg. The network was designed using the allocated IPv4 address block 172.30.34.0/23 and incorporated VLAN-based segmentation to separate departments and improve network organisation.

The implementation included VLAN configuration, trunk links, inter-VLAN routing and an IP addressing plan for the different departments and network services. Network Address Translation (NAT) was configured to support communication between internal network addresses and external networks. A second ISP connection was also incorporated to address the requirement for improved connectivity resilience.

Remote network management was addressed by configuring SSH version 2 on the R-EDGE router. Testing confirmed that SSH was enabled and that a remote session could be established from MGMT-PC1 through VLAN 40 (MANAGEMENT). The router interfaces were also checked, and selected connectivity tests were performed.

The recorded test results provide evidence of progress towards the project requirements. However, successful NAT operation, automatic ISP failover, dedicated management access through VLAN 70 (NET-MGMT), and access restrictions for unauthorised hosts must be supported by their respective test results before they can be considered fully verified.

Overall, the project demonstrates the application of network segmentation, routing, address translation, ISP redundancy and secure remote administration in a simulated business network. The Cisco Packet Tracer file, technical report, testing evidence and GitHub portfolio should be submitted together to document the design, implementation and verified results.